Chichester City LCWIP.

Appendix B Cycling audit & route assessments

V1 June 2020 V2 Feb 2021 # © Transport Initiatives LLP 2021 All photos © Transport Initiatives except where stated All OS mapping © Crown copyright and database rights 2021 Ordnance Survey 100018803

1.	Introduction

To assess how safe and convenient it is to cycle around Chichester, a desk-based study was carried out to assess the level of cycling skills needed to use the highway network. This was followed up by a number of site visits to confirm the desk research and investigate crossing points on the network.

The process was based on Transport Initiatives’ Cycle Skills Network Audit, scaled back for speed and cost-effectiveness (omitting an area-wide assessment of paths and cycle tracks). Detailed plans of the audit are given below, followed by assessments of the identified routes. These were revised following public consultation in late 2020, also taking into account new Government cycle design guidance LTN 1/20 published in July 2020.

Family cycling, South Street

NCN 2 path between Chichester station and Chichester College

2.	Cycle audit plans

Table 1 sets out the Bikeability assessment levels used in the following plans.

These are based on the Cycle Skills Network Audit (CSNA) process developed by Transport Initiatives. This has been carried out in many councils across the UK over the last ten years.

Table 1: Bikeability assessment audit levels

| Level | Type | Description |
| --- | --- | --- |
| Level 2 | Road | Residential or other quiet street, suitable for most people cycling including older children (i.e. with skills equivalent to Level 2 Bikeability) |
| Private | Road | Private street – access may be allowed at some times (generally similar to Level 2) |
| Level 3 (less busy/ rural) | Road | Busier road in urban areas (e.g. rat run) or minor road in rural areas with lower traffic but high speeds, generally only suitable for less risk averse cyclists |
| Level 3 (busy) | Road | Busy road only suitable for less risk averse cyclists (i.e. with skills equivalent to Level 3 Bikeability) |
| Beyond Level 3 | Road | Very busy road with fast moving traffic, unsuitable even for experienced cyclists (e.g. A27) |
| Steps | Crossing | Grade-separated crossing (bridge or subway) with steps |
| Ramp | Crossing | Grade-separated crossing with ramp but cycling prohibited |
| Level 1 | Crossing | Grade-separated crossing with ramp with cycling allowed |
| Level 2 | Crossing | Higher quality/protected crossing – walking only |
| Level 2 – cycles | Crossing | Higher quality/protected crossing – walking & cycling (or cycling-only) |
| Level 2 | Crossing | Lower quality/unprotected crossing – walking only |
| Level 2 – cycles | Crossing | Lower quality/unprotected crossing – walking & cycling (or cycling-only) |
| Beyond Level 3 | Crossing | Very hazardous crossing for any user |

Plan 1: Bikeability assessment of roads and crossings in the LCWIP area

Plan 2: Bikeability assessment of roads and crossings in the Chichester city area

Plan 3: Bikeability assessment of roads and crossings in Chichester city centre

Plan 4. Crossings

Plan 5. Crossings (north-west area)

Plan 6. Crossings (north-east area)

Plan 7. Crossings (south-east area)

Plan 8. Crossings (south-west area)

Table 2. Schedule of Crossings

| Ref | Crossing type | Level | Gateway | Comments |
| --- | --- | --- | --- | --- |
| CC001 | Toucan phase | 2.1 | Existing shared |  |
| CC002 | Toucan phase | 2.1 | Existing shared | Adequate for pedestrians but the intention is that southbound cyclists turn into very sub-standard cycle lane so Level 3 for cyclists |
| CC003 | Refuge | 3.1 | Existing pedestrian & potential shared |  |
| CC004 | Cycle only refuge into cycle lane | 3.1 | Potential cycle | Lane is narrow & lost in vegetation |
| CC005 | Footbridge | 0.5 | Existing pedestrian & potential shared |  |
| CC006 | Under bridge | 1 | Existing shared |  |
| CC007 | Footbridge | 0.5 | Existing pedestrian & potential shared |  |
| CC008 | Refuge | 2 | Existing pedestrian & potential shared |  |
| CC009 | Puffin | 2 | EP |  |
| CC010 | Dropped kerbs | 3.1 | Potential shared |  |
| CC011 | Footbridge | 1 | Existing shared |  |
| CC012 | Dropped kerbs | 4 | Potential shared | Awful |
| CC013 | Dropped kerbs | 4 | Potential shared | Awful |
| CC014 | Island | 4 | Potential shared | 5 lanes of fast moving traffic to cross |
| CC015 | Reservation | 4 | Potential shared | It may be wide but it is still very hazardous getting to it |
| CC016 | Refuge | 3 |  | Narrow & no tactile |
| CC017 | Refuge | 3 |  | Narrow & no tactile |
| CC018 | Puffin | 2 | Existing pedestrian & potential shared |  |
| CC019 | Puffin | 2 |  |  |
| CC020 | Refuge | 3 |  | Narrow |
| CC021 | Toucan phase with reservation | 2.1 |  |  |
| CC022 | Cycle only phase | 2.1 |  | Pointless as you need level 3 skills to get to it along the very hazardous Shopwyke Road |
| CC023 | Puffin with reservation | 2 | Existing pedestrian & potential shared |  |
| CC024 | Toucan | 2.1 | Existing shared |  |
| CC025 | Dropped kerbs | 3.1 |  |  |
| CC026 | Island | 3.1 |  |  |
| CC027 | Dropped kerbs | 3.1 |  |  |
| CC028 | Island | 3 |  |  |
| CC029 | Reservation | 3.1 |  | All this roundabout should be tightened up |
| CC030 | Island | 2.1 | Existing shared |  |
| CC031 | Island | 2.1 | Existing shared |  |
| CC032 | Island | 2.1 | Existing shared |  |
| CC033 | Island | 2.1 | Existing shared |  |
| CC034 | Island | 2.1 | Existing shared |  |
| CC035 | Island | 2.1 | Existing shared |  |
| CC036 | Puffin | 2 | Existing pedestrian |  |
| CC037 | Toucan | 2.1 | Existing shared |  |
| CC038 | Pelican | 2 | Existing pedestrian & potential shared |  |
| CC039 | Refuge | 3 |  | Narrow |
| CC040 | Refuge | 3 | Potential shared | Narrow & no tactile |
| CC041 | Refuge | 3 |  | Narrow & no tactile |
| CC042 | Refuge | 3 |  | Narrow |
| CC043 | Puffin | 2 | Existing pedestrian & potential shared |  |
| CC044 | Refuge | 3 |  | Steps on east side render this redundant for other than access to bus stop |
| CC045 | Toucan | 2.1 | Existing shared |  |
| CC046 | Island | 3 |  | Alignment of dropped kerbs is terrible to accommodate hazardous cycle give way & puts all users at more risk |
| CC047 | Island | 3 |  | Only access from east side of St Pauls Road to centre of gyratory. Angle of crossing awful & signs in island obstruct badly |
| CC048 | Island | 3 |  | No tactile & no proper dropped kerb on south side where pedestrians are pitched into a bus stop |
| CC049 | Island | 2 |  | Not great |
| CC050 | Subway | 0.5 | Existing pedestrian & potential shared |  |
| CC051 | Reservation | 3 |  | Two lanes of fast approaching traffic. No tactile |
| CC052 | Island | 2 |  | No tactile |
| CC053 | Island | 3 |  | Poor sightlines on south side |
| CC054 | Toucan | 2.1 | Existing shared |  |
| CC055 | 3 way island | 2 |  | No tactile on north/south arm |
| CC056 | 3 way island | 2 |  | No tactile |
| CC057 | Puffin | 2 |  |  |
| CC058 | Puffin | 2 |  |  |
| CC059 | 3 way island | 2 |  | No tactile |
| CC060 | Puffin | 2 | Existing pedestrian & potential shared |  |
| CC061 | Refuge | 3 |  | Narrow & no tactile |
| CC062 | Puffin | 2 | Existing pedestrian & potential shared |  |
| CC063 | Refuge | 3 |  | Narrow & no tactile |
| CC064 | Pelican | 2 | Existing pedestrian & potential shared |  |
| CC065 | Island | 3 |  | Wide fast approaches |
| CC066 | Island | 3 |  | Wide fast approaches |
| CC067 | Island | 3 |  | Wide fast approaches |
| CC068 | Island | 3 |  | Wide fast approaches |
| CC069 | Pelican | 2 |  |  |
| CC070 | Refuge | 3 |  | Narrow & no tactile |
| CC071 | Refuge | 3 |  | Narrow & no tactile |
| CC072 | Refuge | 3 |  | Narrow |
| CC073 | Refuge | 3 |  | Narrow |
| CC074 | Toucan | 2.1 | Existing shared |  |
| CC075 | Puffin | 2 | Potential shared | Potential only if changed & moved to desire line by junction |
| CC076 | Raised junction | 3 | Potential shared |  |
| CC077 | Raised table | 3 |  |  |
| CC078 | Raised junction | 3 | Potential shared |  |
| CC079 | Raised junction | 3 | Potential shared |  |
| CC080 | Raised junction | 3 | Potential shared |  |
| CC081 | Raised table with build out | 3 | Potential shared |  |
| CC082 | Raised table with build out | 3 |  |  |
| CC083 | Raised table with build out | 3 | Potential shared |  |
| CC084 | Raised table with build out | 3 | Potential shared |  |
| CC085 | Footbridge | 0 | Potential shared | Steps only |
| CC086 | Footbridge | 0 | Potential shared | Steps only |
| CC087 | Footbridge | 0.5 | Existing pedestrian & potential shared |  |
| CC088 | Raised table with build out | 3 |  |  |
| CC089 | Raised table with build out | 3 |  |  |
| CC090 | 3 way pelican with island | 2 | Existing pedestrian & potential shared |  |
| CC091 | Puffin with reservation | 3 | Existing pedestrian & potential shared |  |
| CC092 | Cycle gaps | 3.1 | Potential cycle | Awful |
| CC093 | Toucan | 2.1 | Existing shared |  |
| CC094 | Island | 3 |  | To/from island. Poor sightlines. Should be signals |
| CC095 | Puffin with island | 2 | Existing pedestrian & potential shared | Staggered |
| CC096 | Toucan | 2.1 | Existing shared |  |
| CC097 | Refuge | 3 |  | Narrow & no tactile |
| CC098 | Puffin | 2 |  |  |
| CC099 | Raised junction | 3 | Potential shared |  |
| CC100 | Raised table | 3 |  |  |
| CC101 | Footbridge | 1 | Existing shared | Railway, Westgate |
| CC102 | Subway | 1 | Existing shared | A27, Fishbourne |
| CC103 | Bridge | 1 | Existing shared |  |
| CC104 | Subway | 1 | Existing shared | Centurion Way, Lavant |
| CC105 | Subway | 1 | Existing shared |  |
| CC106 | Puffin | 2 | Existing pedestrian & potential shared |  |
| CC107 | Footbridge | 0 | Potential shared | Steps only |
| CC108 | Island | 3 | Potential shared | Tapers narrow & wide approaches. No tactile |
| CC109 | Island | 3 | Potential shared | Tapers to narrow & wide approaches. No tactile |
| CC110 | Island | 3 | Potential shared | Wide fast approaches & no tactile |
| CC111 | Reservation | 3 |  | Wide fast approaches & no tactile |
| CC112 | Reservation | 3 |  | Wide fast approaches & no tactile |
| CC113 | Toucan | 2.1 | Existing shared |  |
| CC114 | Island | 3 |  | Narrow & no tactile |
| CC115 | Island | 2.1 | Existing shared |  |
| CC116 | Island | 3 | Potential shared | Narrow & no tactile |
| CC117 | Island | 3 | Potential shared | Narrow & no tactile |
| CC118 | Puffin | 2 | Existing pedestrian & potential shared |  |
| CC119 | Puffin | 2 | Existing pedestrian & potential shared | Not quite on desire line |
| CC120 | Semi raised junction | 3 | Potential shared | Only raised & not much on one side |
| CC121 | Island | 3 | Potential shared | Two lane fast approaches. No tactile |
| CC122 | Pedestrian phase with refuge | 2 |  | No tactile in refuge |
| CC120 | Pedestrian phase | 2 |  |  |

Only 43% of crossings are Level 1 or 2 (including cycle crossings). This is very low compared to other areas studied in the UK.

Plan 9. Cycle routes (blue) and Rights of Way (green)

3.	Route proposals & RST assessments

The plans below show the proposed interventions for the routes to be promoted by CDC, with RST (Route Selection Tool) assessments (based on the proposals in the initial LCWIP). There is no assessment for the town centre core zone as this contains individual links rather than defined routes.

Plan 10. Proposed cycle routes (inset shows Route R continuing to north)

Plan 11. Proposed interventions in core zone

Route A – Lavant

Plan 12. Proposed interventions – Route A

Route B – University

Plan 13. Proposed interventions – Route B

Route E – Vinnetrow

Plan 14. Proposed interventions – Route E

Route F – North Mundham

Plan 15. Proposed interventions – Route F

Routes G – Chichester Canal & H – Stockbridge

Plan 16. Proposed interventions – Routes G & H

Route K – Westgate

Plan 17. Proposed interventions – Route K

Route N – St Pancras

Plan 18. Proposed interventions – Route N

Route Q – College

Plan 19. Proposed interventions – Route Q